Release Notes 3.01.11 intern

# Feature

part set via zone, key switch part set

Key Switch Part Set Momentary

Key Switch Part Set Latched

IT/IP Security

latest mbedTLS library

Zone type: Set only, Part Set only

it needs a zone type for home automation or access control to set/part set the system only.

Part set is realized via zone attribute called “part set” (similar to part set at a key switch zone)

attribute:

Part set = Off -> set is possible only

Part set = On -> part set is possible only

This zone type is available for radio zones, wired zones and I/O-module zones.

Scheduler Set/Unset

Replaced Calendar Sets with a new version (table view). Config saved from previous software versions will be converted to the new scheme format

Note that this mainly affects GUI, WBI and config data. The underlying scheduling code remains unchanged (e.g. events, exceptions)

# Change Requests:

Configuration Print improvement

Outputs with more details

RF repeater with selected components

Communication with more details

all relevant system settings

Special CID/SIA events for window sabotage and class break

Log file export via WBI

Display text now included in the printout

IP Settings, network setup

IP settings are now also loaded as stored in the config

General Fault trigger on A/C fail (VDS)

General Fault output now triggers on A/C fail after delay expired, rather then immediately (VDS compliance).

Camera time out, IP zone missing

Supervision Cam  On/Off

response time IP-zone selectable

telecoms Priority and trouble within public network, comms priority

Reduced ARC and social care call attempts from 15 to 3.

Speech calls now max 3 attempts per recipient but each recipient is tried once on each available path before moving on to next attempt.

SMS calls now max 3 attempts per recipient on path 1 and then repeat for path 2 (for each unsuccessful call on path 1).

Also improved "Comms Trace" messages.

# Bugfix

control device

button C, outputs, user code

It needs always a user code (or Prox) in order to switch an output via control device similar to unset panel.

entering user code (or prox) + pressing button C -> OP xyz switches

pressing button C only -> OP xyz doesn’t switch

Control device can display “Trouble”.

Reason:

The control device (button C) can also be used to switch safety-relevant outputs.

control device, button C, outputs, without user code, status panel

correct trouble message

Add new user at WBI and user related devices prox,remotes, pendants

Note regarding EN 50131 requirements:

Within the first routine (admin user log-in) the admin use is able to create a new user with adding prox, remotes and pendants.

Within a second or later routine (admin user log-in) the admin user can only change name, type and partitions from this new other user.

IP finder fix

Finding the Secvest with the IP Installer tool in the network

mobile phone modules and SIM PIN 0000

|  | FUMO50000 | FUMO50001 | ESMO50000 |
| --- | --- | --- | --- |
|  | Huawei | SierraWireless | SierraWireless |
|  | MG 323-B | HL 6528 | HL7692 |
| SIM PIN = 0000 | OK (Fixed) | Ok | OK |
| SIM PIN request Disabled | OK | OK | OK |

Push and IP-Gateway

Push notification only worked if PPP connection already active for another service (e.g. email).

i.e. at IP gateway

Ethernet

that's clear, only ethernet

IP Mobile - Ethernet

At least e-mail must be activated, but no e-mails need be sent (configure accordingly)

Set up ABUS Server, App and Push via Ethernet.

Ethernet - IP mobile

At least e-mail must be activated, but no e-mails need to be sent (configure accordingly) and

Set up ABUS Server, App and Push via Ethernet.

There must be an Ethernet fail for the IP stack to switch to IP-Mobile.

IP Mobile

At least e-mail must be activated, but no e-mails need be sent (configure accordingly)

Set up ABUS Server, App and Push via Ethernet.

outdoor sounder test menu WBI

It was possible to edit and delete the Outdoor sounder via Test Menu.

After klicking the line it doesn’t appear edit screen.

It is not possible to edit (name and partitions) and to delete the sounder anymore within WBI user and WBI installer.